Topic 17: Acoustic and Ultrasonic Leak Detection for Industrial Systems

- Motivation / Problem Statement

Long-term operation causes:

- corrosion

- cracks

- mechanical damage

- leakage problems

Pipeline leaks can lead to:

- environmental pollution

- safety risks

- economic losses

MEHODS:

- Principle of Acoustic Emission (AE) Leak Detection

- Leakage creates acoustic waves because fluid escapes through a hole under pressure.

- These waves propagate inside the pipeline.

- AE sensors installed on the pipeline surface capture these signals.

- The leak position can be calculated by comparing the arrival times of waves at different sensors.

Challenges in Acoustic Leak Detection

Main difficulties:

- AE signals contain:

  - useful leak information

  - environmental noise

  - vibration disturbances

- Noise reduces localization accuracy.

- Principle of Ultrasonic Leak Detection

- When high-pressure gas escapes through a crack or hole:

  - pressure changes occur

  - turbulence is generated

  - ultrasonic waves are produced

- These ultrasonic waves can be detected remotely without physical contact.

Advantages of Ultrasonic Detection

- Non-contact measurement

- Non-destructive testing

- Suitable for:

  - industrial pipelines

  - pressure vessels

  - outdoor installations

- Can detect leaks from a distance.

Uses only:

- 2 ultrasonic sensors

One sensor:

- scanning sensor

Second sensor:

- reference sensor

WORKING FREQUENCY RANGES:

20–80 kHz ultrasonic range gas leak detection

80–200 kHz AE range pipeline structural/leak monitoring

LITERATURE REVIEW:

- Acoustic and ultrasonic leak detection relies on capturing pressure-generated sound waves produced during leakage.

- AE methods are effective for real-time pipeline monitoring.

- Signal processing is essential because industrial environments contain strong noise.

- MED improves leak signal extraction by removing noise.

- TDOA methods allow leak localization using multiple sensors.

- Ultrasonic imaging methods provide non-contact leak detection.

- Virtual sensor arrays reduce hardware requirements while maintaining localization accuracy.

- Modern approaches combine:

  - signal filtering

  - frequency analysis

  - beamforming

  - machine learning techniques

DIFFERENCE:

| Method | Acoustic Emission | Ultrasonic Detection |
| --- | --- | --- |
| Sensor type | AE piezoelectric sensors | MEMS microphone |
| Contact | Usually contact | Non-contact |
| Frequency | ~80–200 kHz | 20–80 kHz |
| Application | Pipeline health monitoring | Remote leak detection |
| Signal processing | ML + feature extraction | FFT + filtering |
| Advantage | High sensitivity | Long-distance detection |

| Sensor | Paper | Frequency | Application |
| --- | --- | --- | --- |
| AE Piezoelectric Sensor (R15I-AST) | Pipeline leakage detection | 80–200 kHz | Pipeline leak monitoring |
| MEMS Ultrasonic Microphone | Ultrasonic leak detector | 20–80 kHz | Gas leak detection |
| Parabolic reflector + microphone | Ultrasonic detection | 20–80 kHz | Long-distance leak detection |
| Conical horn + microphone | Ultrasonic detection | 20–80 kHz | Directional leak sensing |